Part II Speed of Light In Fiber Optic Cable

Setup

- Turn on oscilloscope.

- Set controls as follows: Vertical Switch – both, Trigger Mode - Auto   Trigger source - Ch1   Trigger Slope – Positive, Ch 1 volt/div  = 1, Ch 2 volt/div = 0.5, Input Coupling - AC, Time/div – 50 ns, Add/Alt/Chop – Alt

- Connect probe between Ch1 and blue Reference point on circuit board. Connect probe ground to ground point near Reference point.

- Connect probe between CH 2 and Delay point and probe ground to nearby ground point on board.

- Set CH 2 input selector to GND.

- Plug in adapter and connect to jack on circuit board. Yellow LED on board should be on.

- Turn the Calibration Delay knob to 12 o’clock position. Connect short (~ 15cm) piece of fiber optic cable between D3 and D8 and tighten nuts.

Calibration

- Make sure probe is in 1X position if 1X/10X type. A pulse should be visible on CH 1 ( ~ 3.5v high).

- Turn CH 2 input to AC. A second pulse that went through the short cable should be visible (~1v  high).

- Use the positioning controls so Ch2 baseline is on top of Ch1 baseline.

- Adjust positioning controls so Ch 1 peak is on left side. Adjust calibration knob to put CH 2 peak on top of Ch. 1 peak. Use scope magnification controls and lower sec/div to get best fit.

- Turn off magnification and set time/div back to 50 ns (.05 microsec). Remove short cable and put in box.

Measurement & Analysis

- Do the steps below first using 20m cable and then repeat with 40m cable. If you are using the 100 Mhz scope look for a 20/100  button near bottom of front of scope. Use whichever position gives clearest scope display.

- Connect the cable between D3 and D8. Do not change calibration control.

- There should be a higher start pulse on the left and a weaker return pulse to the right. Adjust time/div as needed to separate peaks.

- Measure the number of divisions between peaks to nearest 1/10 division. Multiply by time/div to get time.

- Use length of cable (m) and time between peaks (sec) to get speed of light in cable. Compute index of refraction (book value ~1.49).